Bootrom  -  Alpha-Version 0.1

This bootrom code allows you to boot a PC compatible computer over

a IP network. It adheres strictly to the specifications suggested

by Jamie Honan (see file spec.doc). However, this is alpha version

code, which you are allowed to use and distribute provided that you

adhere to the following rules in addition to the GNU general public

licence:

1.) Do not copy this code to any publicly accessible FTP server, UUCP

server or mailbox. The only place for this code is my own system

and ftp.ix.de and it's mirrors.

2.) Only give this code away to people who are able to recognize code

errors, AND are also able to report them to me via email (address

see below).

Please let me know if you are successful using this code, or if you

have any problems. Either way, please report exactly what kind of

system you are using, network interface type, and a way to repro-

duce the problem or error - and a possible solution or fix if you

find one. Thank you very much in advance for your cooperation, which

is necessary to make this code stable and ready for final release.

Note: This readme only describes how to use and operate this bootrom

code. It does not consider all the other issues involved with

network booting. Again: this is only alpha code, and you should

know the basics of network booting before using it.

Features:

1.) This boot rom uses standard DOS packet driver binaries, so it

should be no problem to find a suitable driver for your network

interface card. As a starting point you can use the Crynwr packet

driver collection (which you can get from many DOS ftp sites).

2.) Complete implementation of IP/UDP stack, including ARP, ICMP and

routing. This means that you can use different servers for BOOTP

and TFTP, and that you can boot accross gateways.

3.) The BOOTP client can load a vendor extension file via TFTP, so

that you are no longer restricted to the 64 byte limit of RFC951.

4.) Due to implementing Jamie Honans bootrom specification you can

boot every operating system you like, as long as there is a way

of creating a suitable boot image. At present, there exists only

one implementation for the Linux operating system.

5.) Boot either from a real ROM or floppy disk.

Known Problems:

1.) The bootrom code is larger than 16kB. This means that you are

not able to burn it into real EPROMs for older NE2000 cards and

it's clones. Most other cards I know of should allow bootroms

with 32kB.

2.) Some packet drivers (I only found the one for WD80?3) do access

I/O ports which do not exist on the system. With DOS this does

not give any problem. However, with the boot rom code the system

hangs. I don't know why the driver and/or hardware behaves like

this. Anybody have any ideas? To use this code with WD cards any-

way, I have included a patched version of the smc_wd packet driver.

3.) Packet drivers have to be COM-programs. EXE type programs are not

supported yet.

TODO-List:

1.) I'm going to implement a way of compressing the bootrom image with

gzip, and then get it decompressed out of the EPROM at run time.

This should solve the problem with the 16kB limit.

2.) Implementation of a user friendly way to compose a bootrom image

out of all the different binaries. This should work with both DOS

and UNIX, and possibly other operating systems as well.

3.) Support EXE type packet drivers.

How to use the binaries in this distribution:

1.) Change into the utils directory.

2.) Copy your packet driver into this directory.

3a) If you are using a DOS system, edit the file mkimage.bat. You must

edit the line which calls the rpatch program. Insert the file name

of your packet driver and setup the options required by your packet

driver. Also you have to specify which bootrom image you want to

use: floppy.bin is used to create a boot floppy, and rom.bin is

used to create EPROMs.

3b) If you are using a UNIX system, you first have to compile the rpatch

program. Unfortunately the makefile is only for DOS, so you have to

compile by hand, or write your own makefile. However, a binary for

Linux is provided. Then continue as in step 3a, except that you should

edit the shell script mkimage instead of mkimage.bat.

4.) Run mkimage in the current directory.

5a) To create a boot floppy, copy the produced file image.bin onto

a floppy using rawrite (under DOS) or dd (under UNIX).

5b) To create an EPROM, run the file image.bin through the dosum

program. This program is taken from Jamie's original bootrom

packet, and is only available as a DOS binary. Of course you

are invited to port it to UNIX :-))  Please not, that this

program will only create 32kB EPROM images. Your actual EPROM

can be larger, but never smaller than that. After you created

a ROM image with dosum, you can burn the image into an EPROM.

How to recompile the boot rom:

1.) You have to have:

Turbo-C, version 1.0

Microsoft Assembler version 5.1

Microsoft Linker

Microsoft Library manager

Microsoft exe2bin program

GNU-make for DOS

If there is one of these programs missing you will not be able to

recompile the code. You are invited to port it to other configu-

rations, though.

2.) Edit the file config.mk to suit your configuration.

3.) Start GNU-make in the kernel directory. This should produce both

bootrom binaries in the correct place, so that you can go ahead

with creating the final image as described above.

Again please let me know if you are successful using this code, or

if you have any problems (together with a detailed problem description).

My email address is: gero@gkminix.han.de

Gero Kuhlmann