Use Separate INT 0Ch Handler For Remote Debugging

In order to further distinguish Stack Faults from IRQ4 remote

debugging, I've changed SWAT to use a separate interrupt handler for

each.  The IRQ4 handler gets installed first, then the Stack Fault

handler iff TRAPSTACK is specified.  Correspondingly, they are

uninstalled in reverse order.

At the same time, the following minor changes are made:

* Define keystroke Ctl-F4 to toggle the Stack Fault handler from the

IDT.	This also entails defining a new bit to mark the current

state.

* In IDTMAC, because we control whether or not we swap the IDT entry

via an external flag bit, I've removed the FORCE state; we now

always swap the two entries.

* Define bits to mark whether or not Divide Overflow interrupt and

Bound Interrupt have been toggled.

* Fix a bug upon exiting SWATTER, we mistakenly checked for the VM bit

in what turns out to be a copy of CR0 (it's not there) instead of

(correctly) FORW_EFL.

MAKEFILE	 Note dependency of SWAT.ASM on SWAT_LC4.INC.

Note dependency of SWAT_INI.ASM on SWAT_LC4.INC.

SWAT_COM.INC	 In IDTMAC, because we control whether or not we swap

the IDT entry via an external flag bit, I've

removed the FORCE state; we now always swap the two

entries.

SWAT_LC3.INC	 Define bit to mark state of Stack Fault IDT toggle.

SWAT_LC4.INC	 Define new module to hold additional flags.

Define bit to mark state of Divide Overflow IDT

toggle.

Define bit to mark state of Bound Interrupt IDT

toggle.

SWAT.ASM	 Define new flags word LC4_FLAG.

Define keystroke Ctl-F4 to toggle Stack Fault state.

Define action to toggle Stack Fault state.

If we're on a 486 or later CPU, when we're planting

an INT 03h, in case we're dealing with a cached ROM

invalidate the cache before comparing the value.

Fix a bug upon exiting SWATTER, we mistakenly checked

for the VM bit in what turns out to be a copy of

CR0 (it's not there) instead of (correctly)

FORW_EFL.

SWAT_INI.ASM	 When installing Divide Overflow handler, mark as

intercepted via new flag bit.

When installing Bound Interrupt handler, mark as

intercepted via new flag bit.

Use separate variables for old IRQ3 handler for

INT 0Bh.

Use separate variables for old IRQ4 handler for

INT 0Ch.

Delete code which replaces LCLINT0C_FVEC value if

TRAPSTACK specified as we now have separate

handlers.

Install Stack fault handler if TRAPSTACK specified.

SWAT_INT.ASM	 Define separate variables for Stack Fault and IRQ4

handlers.

Define routine to check for IRQ4 in service.

Uncomment commented out code to revive Stack Fault

processing.

SWAT_REM.ASM	 Use separate variables for old IRQ3 and IRQ4

handlers.

At INT xx_ORIG, check for @DEVL_LOAD before checking

for the VM bit because if we enter via Stack Fault

the stack has an error code on it and the VM bit

isn't where we think it is.

Delete commented out code in case TR is in LDT (the

CPU won't let that ever happen).

In LCL_INT0C2, check for IRQ4 vs. other (Stack Fault,

software interrupt).